(Specifier Note:  The purpose of this guide specification is to assist the Specifier in correctly specifying sun control devices and their installation.

The Specifier needs to edit these guide specifications to fit the needs of each specific project.  References have been made within the text of the specification to MasterFormat section numbers and titles. The Specifier needs to coordinate these numbers and titles with sections included for the specific project.

Throughout the guide specification, there are Specifier Notes to assist in the editing of the file.  Brackets have been used to indicate when a selection is required. Contact a Draper, Inc. representative for further assistance with appropriate product selections.

Sun Control Devices/Window Blinds are manufactured to metric dimensions. Nominal imperial dimensions have been included in (_).

Because Omega Blinds can be mounted either inside the building or on the exterior façade, the specifier needs to select the appropriate section number and title.  Utilizing the Division 10 number and title for exterior applications or the Division 12 number and title for interior applications.)

[SECTION 10 71 13 - Exterior Sun Control Devices]

OR

- [SECTION 12 21 00 - Window Blinds]

- Draper, Inc. Omega Blinds

GENERAL

SECTION INCLUDES

[Electrically ] [Manually] operated [exterior ]venetian blinds

ACTION SUBMITTALS

Refer to Section [01 33 00 Submittal Procedures] [Insert section number and title].

Product Data: For each type of shading device, including manufacturer recommended installation procedures.

Shop Drawings: Include shading assembly dimensions, [electrical and control wiring], method of attachment and structural support.

CLOSEOUT SUBMITTALS

Refer to Section [01 78 00 Closeout Submittals] [Insert section number and title].

Operation and Maintenance instructions, including the name, address and contact information of local service provider.

QUALITY ASSURANCE

Source Limitation: Obtain shading assembly from single manufacturer as a complete unit including necessary mounting hardware and accessories.

Mock-up:  Install one complete shading assembly at project site.  Obtain Architect’s approval prior to proceeding with installation of remaining shading assemblies.  Accepted mock-up [may] [may not] remain as portion of final work.

DELIVERY, STORAGE AND HANDLING

Refer to Section [01 60 00 Product Requirements] [Insert section number and title].

Deliver shade assembly in manufacturer’s original, unopened, undamaged containers with identification labels intact.

Store shade assembly in accordance with manufacturer’s recommendations, protecting materials and finishes from damage.

WARRANTY

Manufacturer Warranty: Manufacturer agrees to repair or replace shade assembly components that fail in materials within specified warranty period.

Failures include but are not limited to venetian blind hardware, metal finishes, electronic controls, accessories and motors.

Warranty Period:  2 years from date of Substantial Completion.

PRODUCTS

(Specifier Note: Product information is proprietary to Omega Blinds by Draper, Inc.  If additional products are required for competitive procurement, contact Draper, Inc. for assistance in listing competitive products that may be available.)

MANUFACTURER

Basis of Design:  Omega Blinds by Draper, Inc.; 411 South Pearl Street; Spiceland, IN 47385; Phone 765.987.7999; website www.draperinc.com

Subject to compliance with requirements, manufacturers of products of equivalent design may be acceptable if approved in accordance with [Section 01 25 00 Substitution Procedures] [Insert section number and title].

HARDWARE

(Specifier Note: SELECT the appropriate attachment method for the specific project. Spacing and quantity of brackets is dependent on width of blind, contact Draper for assistance.)

Mounting Brackets

[Top-Fixing]:  Manufacturer standard ‘saddle’ brackets.

[Direct-Fixing]: Manufacturer ‘key-hole’ slot for attaching to bolts or screws attached to building structure.

- (Specifier Note: Face fixing is in addition to either top or direct fixing, contact Draper for assistance in determining required quantity and spacing of angle brackets.)

[Face Fixing]:  In addition [top-] [direct-]fixing, provide additional angle brackets for face fixing of the system to the building structure.

Headrail

Material: Extruded aluminum, alloy 6063-T6, 1.5 mm (1/16 inch) thick

(Specifier Note: Head rail size is dependent upon the drop length of the shading device.  SELECT the small head rail size for drop lengths up to 23 feet 7 inches, and the larger heard rail for drop lengths between 23 feet 7 inches and 32 feet 1 inch.)

Size:  [2-3/8 inch by 2-1/8 inch (60mm by 54mm)] [3-1/8 inch by 2-15/16 inch (80mm by 75mm)].

(Specifier Note: To allow access to the motor, tilt shaft and tape spools for maintenance the underside of the head rail should be open.  The closed head rail is only available for interior applications. )

Head rail to be [open] [closed] on the underside.

(Specifier Note: Sliding couplings are required for connecting multiple shading devices to a single motor.)

Internal drive shaft of extruded aluminum tube [with sliding couplings at ends]

Louvers

(Specifier Note: SELECT louvers based on desired aesthetic.  DELETE perforation options if slats are solid.)

Flexible Crowned Louvers: [150 mm (6 inches)] [100 mm (4 inches)] [80 mm (3 inches)] [60 mm (2-1/2 inches)] [50 mm (2 inches)]

Perforations

Openness factor: [3.8 percent] [6 percent] [9.2 percent]

Pattern: [Half] [Highway] [Full]

(Specifier Note:  Coordinate perforation options for the Rolled Edge Louvers with Draper.)

Rolled Edge Louvers: [80 mm (3 inches)] [60 mm (2-1/2 inch) ]

Tape:  Manufacturer standard

Lifting tape:  Manufactured from terylene, tear resistant to 110 daN (225 lbf).

Ladder tape: Manufactured from terylene, with cross webs.  Tape attached at each slat to ensure no movement. Tape mechanically attached at top and bottom of slats

Bottom Rail:  Extruded aluminum, 2 mm (3/32 inch) thick minimum, with [aluminum] [plastic] end pieces.

(Specifier Note: Match size of bottom rail to size of slat.)

Size:  [150 mm by 24 mm (6 inches by 1 inch)] [100 mm by 20 mm (4 inches by 7/8 inch)] [80 mm by 20 mm (3 inches by 7/8 inch)] [60 mm by  20 mm (2-1/2 inches by 7/8 inch)] [(50 mm by 20 mm) 2 inches by 7/8 inch]

Side Guides

(Specifier Note: Perlon cables provide a more transparent look.)

[Perlon Cables] [PVC coated stainless steel cables]: 3-4 mm  (1/8 inch) diameter with tensile strength of 320daN (720 lbf). Cables fixed to the building structure by extruded aluminum bracket or a special clamping bolt.

(Specifier Note: Extruded aluminum side channels can only be used with 60mm (2-1/2 inch) and 80 mm (3 inch) rolled edge slats.)

Extruded aluminum side channels with plastic infill.

(Specifier Note: Head Boxes are optional accessories. Exterior shading devices must be protected in the raised position.  Head Boxes are required if pockets are not designed as part of the building structure.  SELECT 1.5 mm thickness for drops up to 7200 mm (23 feet 7 inches) and 2.0 mm thick for drops for drops of 7200 mm (23 feet 7 inches) up to 9800 mm (32 feet 1 inch).)

Head Box: Custom fabricated from [1.5 mm (0.059 inch)] [2.0 mm (0.079 inch)] thick aluminum, to protect device in raised position.

OPERATOR

Electric Operator to provide raise, lower, and tilt functions.

Motor equipped with a disconnect plug, concealed in aluminum head rail. Motor shall incorporate output shafts to allow the motor to be located in the center of the head rail with drive shafts extended in both directions.

Standard Motor: 110V AC, single-phase, 60 Hz, thermally protected, with torque rating as required by manufacturer.

Motor speed range:  20 to 35 RPM

Amperage range: 1.21 to 1.80

Motor to incorporate push button limit switches in addition to separate push button safety cut off switch to prevent motor running beyond upper limit.

(Specifier Note: Crank operator can only be used for interior applications.)

Crank Operator to rise, lower, and tilt functions.

Gear box: Concealed in headrail.

Incorporate: [fixed] [removable] crank handle for operation of the window blinds.

FINISHES

Slats: Double Stove Enameled

Color: [RAL 9006 (Aluminum)] [RAL 9016 (Traffic White)] [RAL 9007 (Grey Aluminum)]

Other exposed aluminum components: [clear anodized] [Polyester powder coated to match the slats]

Concealed aluminum components:  mill finish

Factory finishes are to be uniform, smooth and without blemishes.

CONTROLS

(Specifier Note: SELECT controller based on desired function.  BC2 controller will operate sun control in upward and downward direction upon activation. SFBC2 controller will operate sun control based on sun levels and preset stops.)

External Controller:  [BC2 - 2 motor building controller, no sun tracking, C156.188] [SFBC2 – 2 motor solarflex building controller, C156.189].

(Specifier Note: SELECT operator based on desired function.  Wall switches operate sun control in groups, to presets.  The Sensor and Control System require an interface with an outside system or sensor.  The BACnet Router provides separate zones of control, enabling BACnet communication to work more effectively.

Peripheral Operator:  [Gen 3 Wall Switch] [Sensor and Control System Interface, C232.008] [BACnet Routner, C103.086]

(Specifier Note: In addition to the operator and controller, SELECT desired sensors or DELETE in their entirety. Brightness sensors look at the brightness in the sky, Directional sensors look at a specific point in the sky.  The Wind Sensor provides feedback to the controls based on wind speed and direction.  The Multi Sensor provides feedback to the controller based on multiple information points including brightness, wind and temperature.  Exterior sun control devices require either the wind sensor or the multisensory so that sun control is retracted during wind events.)

Sensors:

Brightness Sensor:  [Directionless, Sun Sensor, C232.007] [Directional, Sun Sensor, C232.006]

Wind Sensor:  C232.005

Multi Sensor:  C232.004

- (Specifier Note: Wind control is required for all exterior applications, to assess conditions an anemometer is required as part of the system.)

Wind control for exterior sun control devices:  Anemometer.

FABRICATION

Fabricate shading devices for custom installation as indicated.

Finished assemblies shall be square, true to size and free from twist or other defects that affect strength and appearance.

EXECUTION

PREPARATION

Verify field dimensions of windows prior to fabrication of shading device.

Coordinate structural requirements of shading device to ensure proper attachment and support.

(Specifier Note: DELETE requirements related to power and motors for manually operated installations.)

Coordinate requirements and location of power supply, conduit, and wiring required for motors and controls.

Test and verify all set motor limits prior to wiring.

INSTALLATION

Comply with shading assembly manufacturer’s written instructions and shop drawings.

TESTING AND DEMONSTRATION

Operate shade through complete cycle of lowering, raising and tilting to ensure proper operation. Correct deficiencies.

Demonstrate operation of shading assembly to Owner's designated representatives.

PROTECTION

Clean and protect shading assembly after installation from damage during construction operations. If damage occurs, remove and replace damaged components or entire unit as required to provide units in their original, undamaged condition.

END OF SECTION