SECTION 11 66 23

GYMNASIUM PICKLEBALL SYSTEMS

***** Draper, Inc. manufactures floor sleeves, pickleball game standards, nets, and other accessories required for competition type pickleball ball installations within gymnasiums. The specifier will need to select the appropriate items and delete non-applicable components and options. *****

PART 1 - GENERAL

1.1	SUMMARY

A.	Section includes: Pickleball equipment for gymnasiums and other interior installations:

***** Edit this list to indicate items being specified. *****

1.	Floor sleeves and cover plates.

3.	Fixed length standards.

4.	Pickleball nets and center tie downs with net strap.

B.	Related sections:

***** List other specification sections dealing with work directly related to this section such as the following. *****

1.	Section 03300 - Cast-in-Place Concrete: Concrete floor slabs and footings to receive floor sleeves.

2.	Section 09640 - Wood Flooring: Layout and painting of court lines to be coordinated with installation of floor sleeves.

3.	Section 09650 - Resilient Flooring: Layout of court lines to be coordinated with installation of floor sleeves.

1.2	REFERENCES

***** List by number and full title reference standards referred to in remainder of the specification section. Delete non-applicable references. *****

A.	ASTM A500 - Formed Welded Seamless Structural Tubing in Rounds and Shapes.

B.	ASTM B85 - Aluminum-Alloy Die Castings.

C.	ASTM B221 - Aluminum-Alloy Extruded Bar, Rod, Wire, Shape, and Tube.

1.3	SUBMITTALS

A.	Submit in accordance with Section 01330 - Submittal Procedures:

1.	List of proposed products and product data.

2.	Shop drawings showing pickleball court layout and floor sleeve locations, dimensions, and method of installation.

3.	Manufacturer's installation instructions.

1.4	QUALITY ASSURANCE

A.	All pickleball equipment, components, and accessories shall be products of a single manufacturer.

B.	Pickleball equipment shall be designed, fabricated, and installed to comply with requirements for competition play of the following associations:

2.	USA Pickleball (USP).

PARTS 2 - PRODUCTS

2.1	ACCEPTABLE MANUFACTURERS

A.	Acceptable Manufacturer: Draper, Inc. Sales offices and manufacturing facility located at: 411 South Pearl Street, Spiceland, Indiana 47385-0425; 765-987-7999.

B.	Manufacturers of equivalent products submitted and approved in accordance with Section 01630 - Product Substitution Procedures.

2.2	MATERIALS

A.	Structural steel tubing: Steel, mechanical, round tubing conforming to ASTM A500.

B.	Extruded aluminum tubing: Schedule 80, ASTM B221, alloy 6063 Temper T5.

C.	Aluminum castings: ASTM B85.

2.3	SLEEVE TYPE PICKLEBALL SYSTEMS

A.	Sleeve Type Pickleball System ( ___ required).

1.	Basis-of-Design: Subject to compliance with requirements, provide Model 502300 Two Pole, Single Court Pickleball System by Draper, Inc. of Spiceland, IN.

2.	Each system consists of one pair of standards, one pickleball net, and center tie down with net strap.

3.	Pickleball standards are 2-3/8" OD 3/16" wall high strength steel mechanical tubing furnished with all necessary hardware. Each post has a sliding lock collar to anchor net and provide proper net tensioning. Sliding collar and post shall be finished with durable gloss black powder coat.

4.	Pickleball net is constructed of #36 nylon 1-3/4” mesh. Double reinforce white vinyl coated nylon headband with enclosed top cable. Bottom and side include nylon reinforced bindings. Net measures 21’ 9” x 36” and is designed for use on a 20’ wide court with standard placed at 22’ 0” inside-to-inside.

B.	Floor Plate, Chrome-plated w/swivel cover (__ required).

1.	Volleyball Cover Plate shall be DRAPER model 501001 Chrome Plated Cover Plate Assembly by Draper, Inc. of Spiceland, IN.

2.	Chrome plated cover plate assembly shall be provided for use in either synthetic or wood floors. Cover plate shall consist of cast aluminum recessed mounting flange, 5-3/16" in diameter, and a 5" diameter chrome plated cover. Cover shall be equipped with a swivel type retaining screw to prevent theft. A special key shall be provided for opening cover. Three #10 x 1-1/2" flat head wood screws shall be provided to securely mount cover plate assembly to floor.

C.	Floor Sleeve Cover Plates (___ required).

1.	Floor Sleeves shall be DRAPER Model 501008 2-3/8" I.D. Floor Sleeve by Draper, Inc. of Spiceland, IN.

2.	Floor sleeve shall be constructed of steel mechanical tubing with an inside diameter of 2-3/8”, welded to a 3" x 3" bottom plate. Sleeve shall be 8-1/2" long and designed to be installed with the bottom at 9 ½" below the playing surface.

3.1	COORDINATION

A.	Coordinate layout of pickleball courts and location of floor sleeves with installation of floor surfacing and application of game lines and boundaries.

B.	Coordinate location of sleeves and required size of sleeve footing with trade responsible for placing concrete. Provide sleeves in adequate time to allow casting in concrete floor slabs. Ensure that setting of sleeve compensates for type of floor finish to be provided.

C.	Ensure that sleeves for each pickleball court are spaced at [22-0"] inside to inside.

3.3	INSTALLATION

A.	Floor cover plates: Install centered directly over floor sleeves in accordance with manufacturer's instructions. Route out floor to ensure cover is flush with finished floor. Install cover with flat head screws.

3.4	FIELD QUALITY CONTROL

A.	Insert standards in sleeves and attach nets and center tie down with net strap. Verify that all items have been provided and are as required for complete installation.

B.	Verify that standards are vertical and rigid. Verify net height settings are accurate.

C.	Provide missing items and correct deficiencies.

3.5	CLEANING AND DEMONSTRATION

A.	Remove protective wrappings and wash surfaces.

B.	Review installed pickleball system with Owner's designated representatives. Demonstrate installation of equipment and operational features.

END OF SECTION