(Specifier Note: The purpose of this guide specification is to assist the Specifier in correctly specifying Exterior Motorized FlexShade® XL motorized roller shading systems and their installation. Exterior Motorized FlexShade XL shading systems are intermediate-sized shades available from 13 to 25 feet wide and up to 15 feet tall. Maximum shade height is limited by cloth weight and shade width, due to their impact on roller deflection, and by cloth thickness because of bundle diameter. Contact Draper® to verify requirements.

The Specifier needs to edit these guide specifications to fit the needs of each specific project. References have been made within the text of the specification to MasterFormat section numbers and titles. The Specifier needs to coordinate these numbers and titles with sections included for the specific project.

Throughout the guide specification, there are Specifier Notes to assist in the editing of the file. Desired options for size, fabric, controls, method of installation and hardware need to be noted. Brackets have been used to indicate when a selection is required. Unless noted otherwise, the first option is the standard feature. Contact a Draper, Inc. representative for further assistance with appropriate product selections.)

SECTION 10 71 13

EXTERIOR MOTORIZED ROLLER SHADING SYSTEM

- Draper®, Inc. Exterior Motorized FlexShade® XL

GENERAL

SECTION INCLUDES

Exterior motorized roller shading systems.

RELATED SECTIONS

Delete any sections below not relevant to this project; add others as required.

Section 061000 “Rough Carpentry:” Blocking for support of window shade hardware.

Section 079200 “Joint Sealants:” Sealants for perimeter of shade system.

Section 084413 “Glazed Aluminum Curtain Walls:” Openings to receive window shades.

Section 084423 “Structural-Sealant-Glazed Curtain Walls:” Openings to receive window shades.

Section 084113 “Aluminum-Framed Entrances and Storefronts:” Openings to receive window shades.

Section 084126 “All-Glass Entrances and Storefronts:” Openings to receive window shades.

Section 085000 “Windows:” Openings to receive window shades.

Section 085113 “Aluminum Windows:” Openings to receive window shades.

Section 085123 “Steel Windows:” Openings to receive window shades.

Section 085200 “Wood Windows:” Openings to receive window shades.

Section 085313 “Vinyl Windows:” Openings to receive window shades.

Section 085413 “Fiberglass Windows:” Openings to receive window shades.

Division 26 - Electrical: Electrical supply, conduit, and wiring for motorized window shades.

REFERENCES

(Specifier Note: COORDINATE references below with test standards specified in Quality Assurance and fabrics specified in Part Two “Fabric” article below. DELETE references that are not project specific.)

American Society of Heating, Refrigerating, and Air-Conditioning Engineers, Inc. (ASHRAE)

ASHRAE 74: Method of Measuring Solar-Optical Properties of Materials.

ASTM International

ASTM D3273: Standard Test Method for Resistance to Growth of Mold on the Surface of Interior Coatings in an Environmental Chamber.

ASTM D6329: Standard Guide for Developing Methodology for Evaluating the Ability of Indoor Materials to Support Microbial Growth Using Static Environmental Chambers.

ASTM E84: Standard Test Method for Surface Burning Characteristics of Building Materials.

ASTM E2180: Standard Test Method for Determining the Activity of Incorporated Antimicrobial Agent(s) In Polymeric or Hydrophobic Materials.

ASTM G21: Standard Practice for Determining Resistance of Synthetic Polymeric Materials to Fungi.

British Standards Institution (BS)

BS 5867-2: Fabrics for Curtains, Drapes and Window Blinds - Flammability Requirements, Specification.

California Code of Regulations (CCR)

California U.S. Title 19 (Division 1, Chapter 8): Fabric and Material Certification.

European Commission (EC)

REACH (EC 1907/2006): European regulation on chemicals and their safe use.

European Union (EU)

Directive 2002/95/EC (RoHS): Directive on the restriction of the use of certain hazardous substances in electrical and electronic equipment.

German Institute for Standardization (DIN)

DIN 4102-1: Fire Behaviour of Building Materials and Elements, Classification of Building Materials, Requirements and Testing.

Ferrari® Group

Texyloop®: Polyester-PVC composite textile recycling initiative.

France Standards, Association Francaise de Normalisation (AFNOR/NF)

NF P92-503: Fire-Resistance of Building Materials and Elements, Electrical Burner Test for Flexible Materials.

NF P92-512: Safety Against Fire. Reaction to Fire Tests. Determination of the durability of the fire reaction classification of materials.

International Oeko-Tex® Association

Oeko-Tex® Standard 100: Globally uniform testing system of textiles for harmful substances which are prohibited or regulated by law, chemicals which are known to be harmful to health, and parameters which are included as a precautionary measure to safeguard health.

International Organization for Standardization (ISO)

ISO 6941: Burning Behaviour of Textile Fabrics, Measurement of Flame Spread Properties of Vertically Oriented Specimens.

Microban® International, Ltd.

Microban® Protection: Antimicrobial protection to inhibit the growth of stain-causing bacteria, mold and mildew.

National Fire Protection Association (NFPA)

NFPA 101: Life Safety Code.

NFPA 701: Standard Methods of Fire Tests for Flame Propagation of Textiles and Films.

Underwriters Laboratories of Canada (ULC)

CAN/ULC-S109: Standard for Flame Tests of Flame-Resistant Fabrics and Films. Method of Fire Tests for Flame-Resistant.

ACTION SUBMITTALS

Refer to Section [01 33 00 Submittal Procedures] [Insert section number and title].

Product Data: For each type of motorized roller shading system, including manufacturer recommended installation procedures.

Shop Drawings: Include opening dimensions if applicable, electrical and control wiring, method of attachment, and structural support.

Samples: Provide fabric and metal finish samples.

Shade Schedule: List rooms, field verified opening dimensions, quantities, type of shade, controls, fabric, and color.

(Specifier Note: Remaining paragraphs are optional fabric requirements. DELETE paragraphs below that are not project specific.)

(Specifier Note: Shade fabrics listed as GREENGUARD® certified qualify as low emitting materials and meet all the requirements of the GREENGUARD Indoor Air Quality or Gold certification program. GREENGUARD certification is optional. COORDINATE the selection of shade fabric paragraph below if GREENGUARD certification is required. DELETE paragraph and sub-paragraph below if not project specific.)

Certificate of Environmental Compliance: Documentation indicating fabrics meet or exceed the following field-validated standards set by UL for products and materials with low chemical and particle emissions for indoor usage.

(Specifier Note: Choose one of two paragraphs below. Second option specifies stricter emission guidelines for the heightened sensitivities of school or other populations.)

GREENGUARD® Certified.

GREENGUARD Gold.

Environmental Certification: Oeko-Tex® Standard 100 Certificate.

Chemical Certification: REACH registration or proof of acceptance.

Lead Free Certification: RoHS labeled or proof of certification.

Recycling Certification: Texyloop® Certificate.

Antimicrobial Protection Certification: Microban® Certificate.

CLOSEOUT SUBMITTALS

Refer to Section [01 78 00 Closeout Submittals] [Insert section number and title].

Maintenance data.

QUALITY ASSURANCE

Source Limitation: Obtain motorized roller shading systems from single manufacturer as a complete unit including necessary mounting hardware and accessories.

(Specifier Note: COORDINATE fabric fire-test-response characteristics paragraph below with fabric specified in Part Two. DELETE paragraph if not project specific.)

Fire-Test-Response Characteristics: Provide shade fabric with the fire-test-response characteristics indicated, as determined by testing identical products per test method indicated by UL or another testing and inspecting agency acceptable to authorities having jurisdiction. Identify materials with appropriate markings of applicable testing and inspecting agency.

Shading system shall be UL listed. Provide documentation and proper labeling.

DELIVERY, STORAGE AND HANDLING

Refer to Section [01 60 00 Product Requirements] [Insert section number and title].

Deliver motorized roller shading systems after building is enclosed and construction where shades will be installed is substantially complete.

Deliver motorized roller shading systems in manufacturer’s original, unopened, undamaged containers with identification labels intact.

Label containers according to Shade Schedule.

(Specifier Note: Draper, Inc. does not warrant against freight damage, concealed or otherwise. RETAIN inspection and storage paragraphs below for all projects.)

Inspect motorized roller shading systems for freight damage, concealed or otherwise, upon delivery to project site. Report damage to freight carrier immediately for replacement of motorized roller window shades.

Store motorized roller shading systems in resealed manufacturer’s original containers.

WARRANTY

Manufacturer’s Hardware and Shade Fabric Warranty: Manufacturer agrees to repair or replace motorized roller shading systems that fail in materials or workmanship within specified warranty period.

Failures include but are not limited to mounting hardware, headbox, and shade fabric.

Warranty Period: 10 years from date of Substantial Completion.

Manufacturer’s Motor and Controls Warranty: Manufacturer agrees to repair or replace motorized roller shading systems that fail in materials or workmanship within specified warranty period.

Failures include but are not limited to controls, electronic accessories and motors.

Warranty Period: 5 years from date of Substantial Completion.

PRODUCTS

(Specifier Note: Product information is proprietary to Draper, Inc. If additional products are required for competitive procurement, contact Draper, Inc. for assistance in listing competitive products that may be available.)

MANUFACTURER

Acceptable Manufacturer: Draper, Inc. Sales offices and manufacturing facility located at: 411 South Pearl Street; Spiceland, IN 47385-0425; Phone 765.987.7999; website www.draperinc.com

Subject to compliance with requirements, manufacturers of products of equivalent design may be acceptable if approved in accordance with [Section 01 25 00 Substitution Procedures] [Insert section number and title].

EXTERIOR MOTORIZED ROLLER SHADING SYSTEM

UL listed, exterior motorized roller shading systems: Electrically operated, motorized, vertical roll-up, fabric shading system. Assembly to include motor operator, controls, mounting hardware and other components necessary for complete installation.

Basis-of-Design: Exterior Motorized FlexShade® XL as manufactured by Draper, Inc.

Roller: 2.364 inch (60 mm) outside diameter aluminum. Roller assembly is integrated with floating bracketry.

Roller Idler Assembly: Plastic idle end insert into roller will fit over steel pin and rotate on pin with use of sealed bearing. Steel pin connects to bearing mounted to bracket mounted inside the headbox. Systems not using sealed bearings are not acceptable.

Fabric Connection to Roller Tube: Fabric panel to incorporate spline attachment at the top and hem pocket at the bottom to allow connection to the hem bar.

Hem bar: Weighted aluminum. [Without] [With] bottom liners (without is standard).

Headbox: Cassette-style extruded aluminum sections.

Size: 5 inches high by 5 inches deep (127 mm x 127 mm) by length required for shade being provided.

Cradle System: Extrusion supports entire roller using cradle system to reduce roller deflection.

Shape: [Angled] [Curved].

Finish: [Textured Slate RAL 7043] [Textured Black RAL 9005] [Textured Sand RAL 1019] [Textured White RAL 9003] [Textured Ivory RAL 1013] [Textured Bronze RAL 7022] [Custom RAL] powder coat finish to meet AAMA 2604 standard for organic coatings.

(Specifier Note: SELECT side channels or cable guides below. Delete the paragraph you don’t want.)

Hem Bar Containment.

Extruded Aluminum Side Channels.

Size: 1-5/8 inches (41 mm) by 1-5/8 inches (41 mm).

Finish: [Textured Slate RAL 7043] [Textured Black RAL 9005] [Textured Sand RAL 1019] [Textured White RAL 9003] [Textured Ivory RAL 1013] [Textured Bronze RAL 7022] [Custom RAL] powder coat finish to meet AAMA 2604 standard for organic coatings.

Guide Cables.

Deck Mount

Wall Mount

Finish: [Textured Slate RAL 7043] [Textured Black RAL 9005] [Textured Sand RAL 1019] [Textured White RAL 9003] [Textured Ivory RAL 1013] [Textured Bronze RAL 7022] [Custom RAL] powder coat finish to meet AAMA 2604 standard for organic coatings.

Electric Operator: Tubular motor concealed inside shade roller tube.

(Specifier Note: SELECT radio technology or wired motor below. Radio technology motor is standard. Delete the paragraph you don’t want. COORDINATE motor selection with controls specified in article below.)

Standard Motor: Sonesse® Radio Technology Motor: 110-120V AC, single-phase, 60 HZ, instantly reversible, lifetime lubricated motor with built-in radio receiver, equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches. (Compatible with Wireless Radio Technology Controls.)

Optional Wired Motor: Sonesse® 110-120V AC, single-phase, 60 HZ, instantly reversible, lifetime lubricated, and equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches.

(Specifier Note: Indicate operator selection as seen from inside the room facing the window. Right hand motor location is standard.)

Location: [Right hand (standard)] [Left hand] [As scheduled]

(Specifier Note: DELETE Motor Controls article not used below.)

RADIO TECHNOLOGY MOTOR CONTROLS

(Specifier Note: SELECT from the control paragraphs below. DELETE paragraphs that are not project specific.)

Individual Control, Group Control and Individual and Group Control:

Decora RTS 1-5 Channel Wall Switch Kit

Situo 1 Single Channel Handheld Remote (indoor only)

Telis 1 Single Channel Handheld Remote (indoor only)

Situo 5 Five Channel Handheld Remote (indoor only)

Telis 4 Five Channel Handheld Remote (indoor only)

Telis 16 Sixteen Channel Remote (indoor only)

Telis 1 Patio Remote

Telis 4 Patio Remote

Eolis RTS Wind Sensor

Soliris RTS Wind/Sun Sensor

Situo 5 Soliris Remote

TaHoma (indoor only)

WIRED MOTOR CONTROLS

(Specifier Note: SELECT from the control paragraphs below. DELETE paragraphs that are not project specific.)

Individual Control

Wall Switch – Toggle three position wall switch.

Wall Switch – Key operated three position wall switch.

Group Control:

ISO relay – ETL component. One ISO relay per motor. Allows 110-120V group switching via toggle switch. Allows for up to 12 motors on one switch.

GC4 – ETL assembly. Panelized version with four ISO relays. Up to three GC4s can be linked together for control of up to 12 motors from any single 15 amp double throw wall switch.

1 Motor Controller. Single motor controller for use with motorized roller shades, Venetian blinds, and louver systems. Can be used as a standalone device or part of a network. When networked, can still be independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. On-board contact closure interface. RS-485 network with built-in network termination. On-board buttons for testing and configuring motors. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 ° F (0 - 60° C).

4 Motor Controller. 4 motor controller for use with motorized roller shades, Venetian blinds and louver systems. Each port is independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. RS-485 network with built-in network termination. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 ° F (0 - 60° C). Dimensions: 3½” x 7” x 1 7/8” (89 x 178 x 48mm). Enclosure: 6" x 4" x 4" (152 x 102 x 102mm).

Individual and Group Control:

IGC4N1. Individual and Group control of up to four motors with up to three intermediate positions per motor.

1 Motor Controller. Single motor controller for use with motorized roller shades, Venetian blinds, and louver systems. Can be used as a standalone device or part of a network. When networked, can still be independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. On-board contact closure interface. RS-485 network with built-in network termination. On-board buttons for testing and configuring motors. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 ° F (0 - 60° C).

4 Motor Controller. 4 motor controller for use with motorized roller shades, Venetian blinds and louver systems. Each port is independently operated and can be configured to respond to wall switches, RF remotes, and signals from 3rd party systems. RS-485 network with built-in network termination. Pluggable terminal blocks. Input power: 110-230VAC @ 50 / 60Hz. Output power: Maximum 6A at input power. Material: ABS plastic. Operating temperature: 32 - 140 ° F (0 - 60° C).

FABRIC

(Specifier Note: SELECT one Basis-of-Design sub-paragraph from light-filtering or room darkening paragraphs below. For further information on available shade fabrics, refer to Draper’s website. DELETE fabrics that are not project specific. DELETE descriptive sub-paragraphs if fabric is limited to Basis-of-Design.)

Light Filtering

SheerWeave® Basic 3% by Phifer®. An economical alternative to traditional solar screen fabrics. Fire Rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), NFPA 701 TM#2 (large scale), CAN/CGSB2-4.162-M80. Bacteria and Fungal Resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Average 3 percent Openness. Average Fabric Thickness: .025 inch. Average Fabric Weight: 16.4 ounces per square yard.

SheerWeave® Basic 5% by Phifer®. An economical alternative to traditional solar screen fabrics. Fire Rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), NFPA 701 TM#2 (large scale), CAN/CGSB2-4.162-M80. Bacteria and Fungal Resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. Average 5 percent Openness. Average Fabric Thickness: 0.022 inch. Average Fabric Weight: 14.1 ounces per square yard.

SheerWeave® Series SW2000 by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 14.26 oz/sq yd, .019 inches thick. Series SW2000 average 5 percent open.

SheerWeave® Series SW2100 by Phifer®: VOC Emissions: GREENGUARD Gold®. Manufacturer to supply GREENGUARD Gold 500 denier fiberglass, vinyl coated and woven into a 2 by 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 14.26 oz/sq yd, .019 inches thick. Series SW2100 average 10 percent open.

SheerWeave® Series SW2500 by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open, .024 inches thick. 16.39 oz/square yard.

SheerWeave® Series SW2500 Performance+ by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave with metalized coating on outward side. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open, .024 inches thick. 16.39 oz/square yard.

SheerWeave® Series SW2600 (2360) by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban ®antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10 percent open, .017 inches thick. 10.4 oz/square yard.

SheerWeave® Series SW2701 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 1 percent open. .027 inches thick. 14.6 oz/square yard.

SheerWeave® Series SW2703 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80, CAN/ULC S-102. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. 3 percent open. .028 inches thick. 14.0 oz/square yard.

SheerWeave® Series SW2705 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. GREENGUARD Gold®. Manufacturer to supply GREENGUARD Gold certificate. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .025 inches thick. 11.8 oz/square yard.

SheerWeave® Series SW2710 by Phifer®: Duplex basketweave fabric—light exterior color combined with dark interior color for thermal comfort and view-through. Fire rating: California U.S. Title 19 (small scale), NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), BS 5867 Part 2 Type B Performance, NFPA 701 TM#2 (large scale), CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4. 162-M80. Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental Certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 10 percent open. .023 inches thick. 10.4 oz/square yard.

SheerWeave® Series SW2900 (2390) by Phifer®: 500 denier fiberglass, vinyl coated and woven into a 2 x 2 basket weave. Fire rating: California U.S. Title 19 (small scale), NFPA 101 (Class A Rating), BS 5867 Part 2 Type B Performance, CAN/ULC-S 109 (large and small scale), CAN/CGSB2-4.162-M80, NFPA 701 TM#1 (small scale), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101 and ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 5 percent open. .017 inches thick. 11.8 oz/square yard.

SheerWeave® Series SW3000 by Phifer®: Vinyl coated fiberglass and vinyl coated polyester woven into 62 by 20 mesh. Fire rating: NFPA 701 TM#1 (small scale), NFPA 101 (Class A Rating), IBC Section 803.1.1 (Class A Rating), NFPA 701 TM#2 (large scale). Bacteria and fungal resistance: ASTM E 2180, ASTM G21, ASTM G22, AATCC30 Part 3, ASTM D 3273, GREENGUARD® Mold and Bacteria Standard ASTM 6329; includes Microban® antimicrobial additives. Environmental certification: Certified to GREENGUARD and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. Safe use: RoHS/Directive 2002/95/EC, US Consumer Product Safety Commission Section 101, ANSI/WCMA A 100.1-2007 for lead content and REACH (EC 1907/2006) compliant. 13.46 oz/sq yd, .028 inches thick. Average 14 percent open.

E Screen™ 1% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .020 inches thick, 13.3 oz/square yard.

E Screen™ 3% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .017 inches thick, 11.6 oz/square yard.

E Screen™ 5% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .016 inches thick, 10.7 oz/square yard.

E Screen™ 10% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. .016 inches thick. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 10 percent open, .020 inches thick, 10.3 oz/square yd.

E Screen™ 1% with KOOLBLACK® by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .021 inches thick. Weight: 12.95 oz/square yard.

E Screen™ with KOOLBLACK®. 3 percent openness factor by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .020 inches thick, 11.3 oz. per square yard.

E Screen™ with KOOLBLACK®. 5 percent openness factor by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .019 inches thick, 10.6 oz/square yard.

E Screen Deco™ 1% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 1 percent open, .022 inches thick. 12.30 oz/square yard.

E Screen Deco™ 3% by Mermet®: PVC coated fiberglass yarn woven in 2 by 2 basketweave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .019 inches thick. 11.50 oz/square yard.

M Screen™ 1% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 1 percent open, .022 inches thick, 13.12 oz/square yard.

M Screen™ 3% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .022 inches thick, 12.27 oz/square yard.

M Screen™ 5% by Mermet®: PVC coated fiberglass in 1 by 2 weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 5 percent open, .022 inches thick, 11.3 oz/square yard.

M Screen Deco™ 3% by Mermet®: PVC coated fiberglass in a rib weave. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 3 percent open, .023 inches thick, 12.721 oz/square yd.

M Screen Deco™ 5% by Mermet®: Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. 5 percent open, .020 inches thick, 11.3 oz/square yd.

T Screen™ 3% by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .025 inches thick, 11.6 oz/square yard.

T Screen Deco™ 1% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .028 inches thick, 13.76 oz/square yd.

T Screen Deco™ 3% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .028 inches thick, 12.98 oz/square yard.

T Screen Deco™ 5% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .028 inches thick, 11.89 oz/square yard.

T Screen™ with KOOLBLACK® 1% by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .024 inches thick, 13.54 oz/square yard.

T Screen™ with KOOLBLACK® 3% by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .029 inches thick, 13.27 oz/square yard.

T Screen™ with KOOLBLACK®. 5 percent openness factor by Mermet®: PVC coated fiberglass woven mesh. Dark on one side, light on the other. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .027 inches thick, 12.48 oz/ square yard.

T Screen Naturals™ 1% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 1 percent open, .027 inches thick, 13.83 oz/square yard.

T Screen Naturals™ 3% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 3 percent open, .027 inches thick, 12.59 oz/ square yard.

T Screen Naturals™ 5% by Mermet®: PVC coated fiberglass satin style. Fire rating: NFPA 701-10 TM#1, California U.S. Title 19, CAN/ULC-S109-03 Small & Large Flame Test. Environmental Benefits: Certified to GREENGUARD® and GREENGUARD Gold® standards for low chemical emissions into indoor air during product usage. RoHS compliant – lead free. Bacterial and fungal resistance: ASTM E2180, ASTM G21. Average 5 percent open, .028 inches thick, 12.24 oz/ square yard.

(Specifier Note: RETAIN paragraphs below with any paragraph above.)

Color: [Match Architect’s sample] [As selected by Architect from manufacturer’s full range] [As indicated on window shade schedule] [Insert color].

Seaming: [None] [Center seams on shade] [Provide seams as shown on Drawings].

Custom Printed Graphics: As submitted to manufacturer in accordance with manufacturer’s guidelines.

EXECUTION

PREPARATION

Verify field dimensions of windows prior to fabrication of motorized roller window shades.

Coordinate blocking and structural support requirements of motorized roller window shades to ensure proper attachment and support.

Coordinate requirements and location of power supply, conduit, and wiring required for motorized roller window shade motors and controls.

Entire length of shipping carton must be supported during storage.

Test and verify all set motor limits prior to wiring.

INSTALLATION

Install motorized roller window shades at locations indicated on [Drawings] [and] [approved Window Shade Schedule].

Provide fasteners appropriate for installation conditions.

Install exterior motorized window shades level, plumb, square, and true. Allow proper clearances for window operation hardware.

Comply with motorized roller window shade manufacturer's written instructions and shop drawings.

Shim shades to correct tracking.

TESTING AND DEMONSTRATION

Test motorized roller window shades to verify that controls, limit switches, [interface to other building systems,] and other operating components are functional. Correct deficiencies.

Demonstrate operation of motorized roller window shades to Owner's designated representatives.

PROTECTION

Clean and protect motorized roller window shades after installation from damage during construction operations. If damage occurs, remove and replace damaged components or entire unit as required to provide units in their original, undamaged condition.

Touch-up, repair or replace damaged products before Substantial Completion.

(Specifier Note: For large projects with several types of shades and fabrics, a window shade schedule may be appropriate. Schedule may be located in this specification section or located on Drawings. DELETE article below if not project specific.)

WINDOW SHADE SCHEDULE

Refer to Drawings for exterior motorized window shade types and locations.

END OF SECTION