(Specifier Note: The purpose of this guide specification is to assist the Specifier in correctly specifying Foundation Extendable Video Wall Mounts and their installation. The Specifier needs to edit these guide specifications to fit the needs of each specific project. References have been made within the text of the specification to MasterFormat section numbers and titles. The Specifier needs to coordinate these numbers and titles with sections included for the specific project.

Throughout the guide specification, there are Specifier Notes to assist in the editing of the file. Brackets have been used to indicate when a selection is required. Unless noted otherwise, the first option is the standard feature. Contact a Draper, Inc. representative for further assistance with appropriate product selections.

AV equipment should be chosen during the planning stages of a project to ensure desired mounting and functions can be achieved. Often parties other than the Designer may select or even provide the AV Displays. Coordination between the mount and the display is critical. An AV consultant or expert either engaged by the designer or the client should be involved whenever possible to facilitate this coordination and ensure the success of the project.)

SECTION 11 52 23

AUDIO-VISUAL EQUIPMENT SUPPORTS

- Draper, Inc. Foundation LCD Display Mounts – Extendable Video Wall Mount

GENERAL

SUMMARY

Section Includes: LCD display mounts that can be extended away from supporting wall for service and access to back of display unit.

ACTION SUBMITTALS

Refer to Section [01 33 00 Submittal Procedures] [Insert section number and title].

Product Data: For each LCD display mount, including manufacturer recommended installation procedures.

Shop Drawings: Include dimensions, method of attachment, structural [and seismic] support.

Coordination Drawings: Provide drawings and information indicating coordination between LCD Displays and display mounts.

DELIVERY, STORAGE AND HANDLING

Refer to Section [01 60 00 Product Requirements] [Insert section number and title].

Deliver Foundation LCD display mounts in manufacturer’s original, unopened, undamaged containers with identification labels intact.

(Specifier Note: Draper, Inc. does not warrant against freight damage, concealed or otherwise. RETAIN inspection and storage paragraphs below for all projects.)

Inspect for freight damage, concealed or otherwise, upon delivery to project site. Report damage to freight carrier immediately for replacement of Foundation LCD display mounts.

WARRANTY

Manufacturer lifetime warranty against manufacturing defects.

PRODUCTS

(Specifier Note: Product information is proprietary to Draper, Inc. If additional products are required for competitive procurement, contact Draper, Inc. for assistance in listing competitive products that may be available.)

BASIS OF DESIGN:

Foundation Extendable Video Wall Mount by Draper, Inc.; 411 South Pearl Street; Spiceland, IN 47385-0425; Phone 765.987.7999; website www.draperinc.com

Subject to compliance with requirements, manufacturers of products of equivalent design may be acceptable if approved in accordance with [Section 01 25 00 Substitution Procedures] [Insert section number and title].

PERFORMANCE REQUIREMENTS

(Specifier Note: Display mounts may require additional bracing to resist seismic loads depending on weight, project location, and installation details. Bracing and other support components are not provided by Draper and will need to be designed for specific applications, and detailed appropriately. DELETE seismic bracing paragraph below if not project specific.)

        - Seismic Bracing: Design LCD display mount support to resist the effects of earthquake motion in accordance with Seismic Design Category [Insert Seismic Design Category] [as indicated on Drawings].

FOUNDATION EXTENDABLE LCD DISPLAY MOUNTS

Foundation Extendable mount that contains a spring-mounted scissor mechanism to hold the display away from the supporting wall for service and access to back of display, without affecting other displays in a multi-display wall.

Distance from wall when not extended: 2-3/8 inch (60 mm) to back of display.

Distance from wall when extended: 8-5/8 inch (220 mm) to back of display.

Fine adjustment to height and depth positioning, allowing edge to edge multi-display mounting.

Mount assembly includes

Standard Wall Rail: extruded aluminum wall rail and two (2) end caps per rail

Rail Length: 31-1/2 inches (800mm)

Finish: Clear Anodized Aluminum]

[[Single] [Two] Multi Display Wall Rail(s): extruded aluminum wall rail and two (2) end caps per rail.

Rail Length: 78-3/4 inches (2000 mm) each

Finish: Clear Anodized Aluminum]

Extendable vertical hanging brackets: 2 per mount.

Finish: Powder coated, black

(Specifier Note: Locking pin is included in mount package. A padlock can be utilized for increased personal safety and security from theft. DELETE the application which is not project specific.)

[Locking Pin] [Padlock]

Display attachment hardware.

Mounting pattern: Suitable for VESA standard: 100-400 mm vertically. No limit horizontally.

(Specifier Note: Display characteristics are critical to a successful display mount installation. The displays are not provided as part of the work result but require coordination especially when displays are to be Owner furnished. The Foundation Extendable Display Mount allows for display weight of up to 99 lbs (45 kg) per mount. The minimum display size to be used with this mount is 37 inches (94 cm) and the maximum is 60 inches (152 cm) Coordinate with the AV Consultant or the Owner to complete the following items for the specific project.)

Display characteristics

Maximum display weight: [Insert display weight] [99 lb (45 kg)] per mount assembly.

Display size: [Insert Display size]

EXECUTION

(Specifier Note: Foundation Extendable Video Wall Mount is meant to be mounted to a wall. COORDINATE requirements with desired installation.)

PREPARATION

Coordinate layout and installation with wall construction and related components penetrating walls such as lighting fixtures.

Coordinate requirements for blocking, structural [and seismic] supports, and bracing to ensure proper installation of video wall mounts and LCD displays.

Coordinate location and requirements for power supply conduit, and wiring required to LCD displays.

INSTALLATION

Install Foundation LCD Display mounts at locations and heights indicated on Drawings.

Utilize necessary hardware, anchors, brackets and fasteners; according to manufacturer's written instructions and as specified.

Foundation LCD Display mounts to be installed square and level, and without gaps between LCD Display.

FIELD QUALITY CONTROL

Verify equipment supports are installed securely to substrate and within tolerances recommended by manufacturer.

Correct deficiencies.

PROTECTION

Protect Foundation LCD Display Mounts after installation from damage during remainder of construction. In the event of damage, remove and replace damaged component or entire unit as required to provide units in their original, undamaged condition.

END OF SECTION