(Specifier Note: The purpose of this guide specification is to assist the Specifier in correctly specifying Foundation® One Mount Systems and their installation. The Specifier needs to edit these guide specifications to fit the needs of each specific project. References have been made within the text of the specification to MasterFormat section numbers and titles. The Specifier needs to coordinate these numbers and titles with sections included for the specific project.

Throughout the guide specification, there are Specifier Notes to assist in the editing of the file. Brackets have been used to indicate when a selection is required. Unless noted otherwise, the first option is the standard feature. Contact a Draper, Inc. representative for further assistance with appropriate product selections.

AV equipment should be chosen during the planning stages of a project to ensure desired mounting and functions can be achieved. Often parties other than the Designer may select or even provide the AV Displays. Coordination between the mount and the display is critical. An AV consultant or expert, either engaged by the designer or the client, should be involved whenever possible to facilitate this coordination and ensure the success of the project.)

SECTION 11 52 23

AUDIO-VISUAL EQUIPMENT SUPPORTS

- Draper, Inc. dvLED Display Mounts – Foundation® One Mount System

GENERAL

SUMMARY

Section Includes: dvLED display mounting system for video walls.

ACTION SUBMITTALS

Refer to Section [01 33 00 Submittal Procedures] [Insert section number and title].

Product Data: For each dvLED display mount system, including manufacturer recommended installation procedures.

Shop Drawings: Include dimensions, method of attachment, and structural support.

Coordination Drawings: Provide drawings and information indicating coordination between LED displays and display mounts

DELIVERY, STORAGE AND HANDLING

Refer to Section [01 60 00 Product Requirements] [Insert section number and title].

Deliver LED display panels in manufacturer’s original, unopened, undamaged containers with identification labels intact.

(Specifier Note: Draper, Inc. does not warrant against freight damage, concealed or otherwise. RETAIN inspection and storage paragraphs below for all projects.)

Inspect for freight damage, concealed or otherwise, upon delivery to project site. Report damage to freight carrier immediately for replacement of LED display mounts.

WARRANTY

Manufacturer lifetime warranty against manufacturing defects.

PRODUCTS

(Specifier Note: Product information is proprietary to Draper, Inc. If additional products are required for competitive procurement, contact Draper, Inc. for assistance in listing competitive products that may be available.)

BASIS OF DESIGN

Foundation® One Mount System by Draper, Inc.; 411 South Pearl Street; Spiceland, IN 47385-0425; Phone 765.987.7999; website www.draperinc.com

Subject to compliance with requirements, manufacturers of products of equivalent design may be acceptable if approved in accordance with [Section 01 25 00 Substitution Procedures] [Insert section number and title].

dvLED DISPLAY CABINET (S)

(Specifier Note: The Foundation Mount System is designed to accommodate LED cabinets from 38-76 mm thick, 600-610 mm wide, and 337.5-343 mm high. Select the LED manufacturer from the list below and delete those not being used. Prior to Specifying - confirm with Draper a current list of displays that can be accommodated)

dvLED Cabinets (s)

- [Insert LED cabinet make and model].

(Specifier Note: Nearly any array size and format can be accommodated – Confirm with Draper. Identify the desired array and DELETE other options)

LED Array Size: [insert array size]

Mount System Wall Mounting Brackets.

Wall attachment brackets to be fabricated from galvanized steel.

Distance from wall surface to front of brackets: 1.584 inches (40.23 mm).

Adjustment minimums and maximums (can vary slightly with LED cabinet model):

X = +/- .25 inches (6.35 mm)

Y = +/- .25 inches (6.35 mm)

Total number of cabinets: [ ______________ ].

LED Cabinet Attachment

LED cabinet shall attach to mounting bracket with LED Mount Brackets of 12 gauge galvanized steel. LED Mount Brackets to include Laser Alignment Notches for use with Laser Levels. Hardware for attaching bracket to cabinet and for x, y, and z adjustment shall be provided by manufacturer.

Adjustment minimums and maximums (can vary slightly with LED cabinet model):

Z = +/- .25 inches (6.35 mm)

Bottom Row: LED Mount Brackets shall attach to four mounting holes on each LED cabinet. LED Mount Brackets to attach to wall bracket support tubes at top and bottom.

All Other Rows: LED Mount Brackets shall attach to upper two mounting holes of each LED cabinet. LED cabinets to attach to wall brackets at top and to LED cabinets at bottom.

LED Mount Brackets to be locked in place on Wall Mounting Bracket Support Tubes with one Vertical Lock per LED Mount Bracket

(Specifier Note: Foundation One Mount System can be specified with or without trim. Select relevant paragraphs below if desired; DELETE if specifying without trim.)

Trim

Formed steel sheet metal attaches to mount structure using neodymium magnets.

Finish: Powder coated black.

Trim shall be expandable to accommodate different LED cabinet thicknesses.

Top and bottom trim shall be slotted to allow for heat dissipation.

Trim Connectors of black plastic shall be provided to connect trim pieces and create a seamless look.

EXECUTION

(Specifier Note: Draper Foundation One Mount System is meant to be mounted to a wall. COORDINATE requirements with desired installation.)

PREPARATION

Coordinate layout and installation with wall construction and related components penetrating walls such as lighting fixtures.

Coordinate requirements for blocking, structural supports, and bracing to ensure proper installation of Foundation One Mount System and LED display panels.

Coordinate location and requirements for power supply conduit, and wiring required for dvLED displays.

INSTALLATION

Install dvLED display mounts at locations and heights indicated on Drawings. Install bottom row of dvLED cabinets first.

(Specifier Note: Hardware for attaching mount system is not supplied by the manufacturer.)

Utilize necessary hardware, anchors, brackets, and fasteners.

dvLED display mount to be installed plumb, square, and level to provide proper alignment of dvLED display cabinets when installed.

Correct deficiencies.

PROTECTION

Protect dvLED display mounts after installation from damage during remainder of construction. In the event of damage, remove and replace damaged component or entire unit as required to provide units in their original, undamaged condition.

END OF SECTION